Suggestions Manual

NEX-GRAPH

Version 3.3 Beta

Overview

Result

Graph Controller

Graph

Analysis

Pass Fail Menu

Tools Menu

Connecting Force Gauge DFS

- Turn on DFS and connect to computer via USB or RS232 cable. (On DFS, and must select Output follow your cable connected.).

OUTPUT MENU

1) USB OUTPUT
2) PS/2 OUTPUT

- Go to “Device > Connect “.

Start Plotting Graph

START Plot Graph		:	Start/Stop plot graph.

AUTO STOP			:	Automatic stop plot-graph when force decreased.

Direction			:	Choose Direction.

Add to Table			:	Keep data to table for analysis.

Save Picture			:	Save graph in picture format (*.png).

Clear Graph			:	Clear graph.

Export Data Logger to Excel	:	Export all point on graph to excel (*.xls).

Export Data Logger to Text File	:	Export all point on graph to text file (*.txt).

  - Choose Compression or Tension, press the “Direction” button.

  - Press the “START Plot Graph” button.

  - When test is finished, press the “Stop” button.

Analysis

Selecting a point on the graph by left click.

  - You can choose to see further detail by selecting a point on the graph line.

Download from DFS’s Memory

Delete Last			: Delete last data to add.

Delete All			: Delete all data.

Download from memory	: Download all data from DFS’s Memory.

Receive from ‘Print’ button	: Wait to receive data from DFS when ‘Print’ button pressed.

Export Data Table to Excel	: Export all data in the table and analysis will send to Microsoft excel automatically.

Export Data Table to text file	: Export all data in the table and analysis will send to text file.

Total				: Show number of data in a table.

- Press ‘Download from memory’.

- All memory on DFS be downloaded to the table.

Analysis, 
the software is calculate from above table.

Receiving data using ‘Print’ Button

- Press ‘Receive from Print button’ and then press ‘Print’ button on DFS.

- The real-time data from DFS will send to table.

Using Pass/Fail Function

ON		:	Turn on Pass/Fail function.

OFF		:	Turn off Pass/Fail function.

Upper		:	If force more then set value, it will say “Upper fail” and row turn red.

Lower		:	If force less then set value, it will say “lower fail” and row turn orange.

- Press ‘ON’ button on Pass-fail function bar.

- Fill up Upper/Lower fields to analyses and result will be green, red or orange.

Export Data Table to Excel

- Go to “File > Export to Excel”.

- All data in the table and analysis will send to Microsoft excel automatically.